Bulk and Retail Petroleum

# Module 12

# Terminology

January 2021

Uncontrolled copy for training purposes only

Terminology

| Absolute Value | The absolute value of a number is the magnitude of that number without considering the positive or negative sign. |
| --- | --- |
| Acceptance Limits of Error | The Acceptance limits of error refers to the limit of error that applies to a device when the performance of the device is tested at the time the class, type or design of that device is examined for approval, at the time the device is inspected prior to its first use in trade, at the time the measuring elements of the device are overhauled or repaired following the failure of the device on inspection to measure within the applicable limits of error, or at any time within 30 days after the time referred to in paragraph (b) or (c). |
| Accuracy | Accuracy is the quality of the process followed to measure a quantity that indicates the closeness of the results compared to the true value. |
| Act | Act, in this Module, refers to The Weights and Measures Act. |
| Adhesion | Adhesion is the tendency of a substance’s molecules to adhere to another substance’s molecules. |
| Analogue Conditions | Ambient conditions are the conditions (humidity, pressure temperature, etc.) of the surrounding atmosphere. |
| Analogue Signal | Analogue Signal is the representation of the magnitude of a variable in the form of a measurement’s physical quantity, which is varied smoothly rather than in discrete steps. |
| API Gravity | API gravity is the amount obtained using the following formula: API Gravity = 141.5 over relative density 60/60°F - 131.5. |
| API Standard | The Manual of Petroleum Measurement Standards was published by the American Petroleum Institute in August 1980 with the designation "API, Standard 2540." |
| ASTM | ASTM stands for “American Society for Testing and Materials.” |
| ASTM-IP | ASTM-IP stands for “American Society for Testing and Materials -- Institute of Petroleum.” |
| ATC | ATC stands for “Automatic Temperature Compensation (or Correction).” |
| Auxiliary Indicating Element | In the case of a rider, a vernier or a digital indicating device, an auxiliary indicating element is the last figure after the decimal sign that is clearly differentiated from other figures ( by size, shape or colour, for example.) |
| BIPM | BIPM stands for “International Bureau of Weights and Measures.” |
| Calibration | Calibration is the comparison of the output of a device with the standard values. |
| Canadian Unit | This unit of measurement is set out in Schedule II to the Act. |
| Capillary Seal | The capillary seal is the thin layer of liquid between moving parts. It reduces slippage through a meter. |
| Cavitation | Cavitation occurs when dissolved gasses found in liquids passing through a pump flash. This causes noises, vibrations and pump inefficiency. This phenomenon is caused when localized pressure goes below Vapour Pressure. |
| Cohesion | Cohesion is the tendency of a substance’s molecules to adhere to each other. |
| Graduated Standard | A graduated standard has a cylindrical shape and, if made entirely of glass, bears graduations on the glass. If made of metal, however, it bears graduations that are adjacent to a glass window in the metal. |
| Dealer | The term dealer designates any person who, in the course of that person's business, sells, consigns, imports, leases or lends devices. |
| Density | The density of a substance is its mass divided by its volume (P = M/V). For gasoline, the density is assumed to be 730 kg/cu. m while the density for diesel is assumed to be 840 kg/cu. m. The metric system was designed so that water will have a density of one gram per cubic centimetre or 1000 kilograms (1 tonne) per cubic metre. |
| Device | A device is considered to be any weight, weighing machine, static measure or measuring machine, including any equipment and accessories attached to, or used in conjunction with, the device that have or can have an effect on the accuracy of the device. |
| Digital-Analogue | Digital/analogue refers to the conversion of a digital signal that varies in discrete steps to one that varies in a continuous manner. |
| Durability | Durability is an instrument’s ability to maintain its performance characteristics over a period of use. |
| EPROM | EPROM stands for "Erasable Programmable Read Only Memory.” |
| EMI | EMI stands for "Electromagnetic Interference.” |
| Filter | A filter is an electronic circuit which by design allows only electrical signals of selected frequency bands to pass with little or no attenuation. |
| Gravity | Gravity is defined as the accelerating tendency of bodies toward the centre of earth. Its symbol is “g." |
| Gross | Gross is the uncorrected registration of the register. |
| Height | With respect to the height of a letter, the height of an upper case letter where words appear in upper case and the height of the lower case letter “o” where words appear in lower case or in a mixture of upper and lower case. |
| Hertz or Hz | Hertz is defined as the unit of frequency. |
| Hydrometer Standard | The hydrometer standard is a local standard that indicates the density or relative density of a fluid. |
| Increment | Increment is the value of the smallest unit that can be indicated or recorded by a device. |
| Industrial Standard | Industrial standards are standards other than those owned by Measurement Canada which are certified in relation to a reference standard and are accurate within the same tolerances applicable to local standards. |
| Initial Inspection | Any inspection performed on an approved device up until the time the device is first verified is considered an initial inspection. |
| Inspection Mark | The inspection mark is a mark made by an inspector after inspection of a device. It consists of a symbol for the time being in use by inspectors and the date of inspection. |
| Inspector | An inspector is a person designated with the title of inspector pursuant to the Department of Industry Act for the purpose of the enforcement of this Act. |
| In-service Limits of Error | In-service limits of error are the limits of error that applies to a device when the performance of the device is tested at any time other than a time referred to in the definition of "acceptance limits of error." |
| LCD | LCD stands for “Liquid Crystal Display.” |
| LED | Light Emitting Diode |
| Linearity of a Scale | The linearity of a scale can be defined as the degree to which it adheres to a straight line from zero load to maximum rated capacity during increased load testing. |
| Load Cell | A load cell is a device which produces an output signal proportional to the applied load. The load cell may use any physical principle included in the field of, but not limited to, electricity, hydraulics, magnetics and pneumatics or combination thereof. |
| Local Standard | A local standard is any standard designated by the Minister under Section 13 of the Weights and Measures Act and certified in relation to a reference standard as being accurate within prescribed tolerances. |
| Machine | A machine is a movable part that has or can have an effect on the accuracy of the machine. |
| Master Meter Standard | Master meter standard is a local standard that registers by mechanical, electrical or electronic means or any combination thereof, in units or multiples or subdivisions of units of volume, the quantity of liquid that passes through the standard during an interval of operation. |
| Master Standard | The master standards, owned by the Measurement Canada, designed and used to calibrate measures and provers. Master standards belong to a class of local standards of higher precision than do measures or provers. |
| Measure or Test Measure | Measure, or test measure, is a narrow neck standard which is filled and emptied by the top. Its capacity does not normally exceed 50 litres. It is generally used to test fuel dispensers. |
| Measurement | Measurement is a set of operations having the object of determining a value of a quantity. |
| Measuring Chamber | A measuring chamber is the portion of a meter within which the measuring occurs. |
| Measuring Element | The measuring element is the part of a meter which, when in operation, displaces a definite quantity of liquid during each cycle. |
| Measuring Machine | A measuring machine is any machine that measures length, area, volume or capacity, temperature or time and/or movable part that has or can have an effect on the accuracy of the machine. |
| Meniscus | The meniscus is the curved surface of a liquid when enclosed by a glass tube. The concave surface is formed by the adhesion of liquid particles to the glass. The convex meniscus of mercury in a glass tube is caused by cohesion between mercury and glass. |
| Metering Assembly | The metering assembly is a device for use in trade for measuring liquids. |
| Metering Installation | The metering installation is the metering assembly and all equipment necessary for the operation of the metering assembly, and includes the piping, pump, valves and reservoir. |
| Metric Unit | Metric unit is the unit of measurement set out in Schedule I to the Act. |
| Metrological Function | Metrological function is the function of a weighing/measuring device that is necessary for the measurement process, including the sensing of the measured quantity, the transmission, processing, storage and correction or adjustment of measurement signals or values and the indication or printing of measurement values -- that can: - impact on the final quantity determination or price calculation; or - affect the validity of transactions. |
| Minister | Minister, in this module, refers to the Minister of Industry Canada. |
| Narrow Neck Standard | A narrow neck standard is a local standard of volume or capacity that has, on or adjacent to the narrow neck of the standard, a mark indicating the volume or capacity of the standard, or that has a machined rim that indicates the volume or capacity of the standard. |
| Net | Net refers to the corrected registration of the register. |
| Noise | Noise can be defined as unwanted signals within an electronic circuit which degrade or modify the desired output or performance. |
| Non-resettable Register | A non-resettable register is a primary register that is intended for long-term use without being reset to zero. |
| OIML | OIML stands for "International Organization of Legal Metrology.” (Organisation internationale de métrologie légale) |
| Pipe Prover Standard | A pipe prover standard is a local standard consisting of a pipe equipped with a piston or ball in which the volume of liquid displaced when the piston or ball traverses all or any part of the length of the pipe is registered by mechanical, electrical or electronic means or any combination thereof. |
| Positive Displacement Meter | A positive displacement meter is a meter which displaces a definite quantity of liquid for each cycle of operation. |
| Positive Seal | The term positive seal refers to the mechanical contact between moving parts, or moving parts and stationary parts, which reduces slippage. |
| Potentiometer | A potentiometer is a resistance unit having a variable or sliding contact which is positioned by the rotation or sliding of a shaft. The motion of the shaft is an indication of that portion or total resistance which is between the contact and each end of the potentiometer. |
| Pressure Loss | Pressure loss is the loss of pressure in a liquid being metered between the inlet and outlet of the meter. |
| Prescribed | The term prescribed signifies that it is prescribed by the regulations. |
| Previous Enactment | Previous enactment refers to any Act of Parliament relating to weights and measures, and any regulations made under that Act, enacted prior to the Weights and Measures Act. |
| Prover or Open Prover | A prover or open prover is a narrow neck standard with a capacity exceeding 50 litres, which is equipped with drain piping and a valve. They are generally used to test meters with a capacity of 115 litres or more per minute. |
| Quadrature | Quadrature, specifically, is any signal, or portion of a signal, whose phase relationship is 90 electrical degrees from the main signal. The term quadrature is commonly used to mean any signal not in the proper phase relationship with the signal to be measured. |
| Quick Acting Valve | A quick acting valve is a 1/4 turn valve. |
| RAM | RAM stands for “Random Access Memory.” |
| Reid Vapour Pressure (RVP) | Reid vapour pressure is the absolute pressure required to prevent liquid from evaporating. For gasoline, it varies from 48.3 - 89.6 kpa or 7 to 13 psi. If we want to pump liquid and not vapour, the minimum pump suction must be greater than the RVP. |
| Reference Pressure | - in the case of a liquid that has an equilibrium vapour pressure of less than 101.325 kPa at measurement temperature, - in metric units, an absolute pressure equal to 101.325 kPa, and - in Canadian units, an absolute pressure equal to 14.73 pounds per square inch; and - in the case of a liquid that has an equilibrium vapour pressure of 101.325 kPa or more at measurement temperature, the equilibrium vapour pressure at the temperature of the liquid as it is being measured |
| Reference Standard | A reference standard is a standard that - represents or registers a unit of measurement referred to in Schedule I or II or that represents or registers a multiple or fraction of such a unit of measurement; - has been calibrated and certified by the National Research Council of Canada; and - is or is to be used as a standard for the purpose of determining the accuracy of a local standard. |
| Register | A register refers to any register that incorporates electronic components in order to process signals that correspond to measured quantities. |
| Registration | Registration refers to a displayed or a printed value. |
| Repeatability | The repeatability test is designed to ensure that the volumetric device will deliver, within prescribed limits, the same volume under the same operating conditions (flow rate, operation sequence, and test quantity). It should be performed when the difference between the result of the fast test and the result of the slow test exceeds the absolute value of the in-service limit of error for the particular test quantity used. |
| RFI | RFI refers to “Radio Frequency Interference.” |
| ROM | ROM refers to “Read Only Memories.” |
| Specific Gravity | Specific gravity refers to the ratio of the weight of a given value of a substance to the weight of an equal volume of a reference substance (usually water at a stated temperature.) |
| SI Units | The term SI units refer to "International System of Units.” (Système international) |
| Static Measure | A static measure is any measure that measures length, volume or capacity and does not have a moving or movable part that has or can have an effect on the accuracy of the measure. |
| Tank | A tank refers to a measuring tank used in trade for measuring the volume of liquid received or delivered, and includes a fixed, portable and vehicle-mounted measuring tank. |
| Tolerance | Tolerance refers to the limits of error. |
| Traceability | Traceability is the property of the result of a measurement whereby it can be related to reference standards, through an unbroken chain of comparisons (Ref. International Vocabulary of Basic and General Terms in Metrology.) |
| Trade | Trade refers to the selling, purchasing, exchanging, consigning, leasing or providing of any commodity, right, facility or service on the basis of measure and includes the business of providing facilities for measuring. |
| Trader | A trader is any person who trades in the course of business. |
| Transfer Point | The transfer point is the fixed point in a measuring assembly where transfer of possession of a quantity of liquid contained in the measuring assembly takes place. |
| Uncertainty | Uncertainty is the value of a measurement result that is expected to lie with stated probability. |
| Vapour Lock | At restrictions in the supply line, the local pressure falls below Reid Vapour Pressure (RVP) and vapour accumulates. The pump must remove this vapour before the liquid can move and this can take some time. |
| VCF | VCF stands for “Volume Correction Factor.” |
| Viscosity | Viscosity is the ability of a liquid to flow. Light body oils have a low viscosity; heavy oils have a high viscosity. |
| Zone of Uncertainty | The zone of uncertainty is the zone between adjacent increments in a digital device in which the value of either of the adjacent increments may be displayed. |