Bulk and Retail Petroleum

# Module 2

# Weights and Measures Act

January 2021

Uncontrolled copy for training purposes only

Table of contents

Section 2	5

Definitions	5

Section 3	5

Approval of Devices by the Minister	5

Section 8	5

Use of Devices	5

Section 10	6

Regulations	6

Section 13	6

Reference and Local Standards	6

Section 15	7

Examination of Devices	7

Sections 19	7

Certificate and marking	7

Section 23	7

Offences and Punishment	7

Section 24	7

Offences and Punishment	7

Section 25	7

Examination within prescribed period	7

Section 26	8

Offences and Punishment	8

Section 28	8

Offences and Punishment	8

Section 30	8

Offences and Punishment	8

Section 31	9

Offences and Punishment	9

Section 35	9

Offences and Punishment	9

Section 36	9

Evidence	9

Section 37	9

Schedule 1	10

Part I – Units Based on the International System of Units	10

Part IV – Customary Units of Measurement with the International System	11

Part V – Prefixes* for Multiples and Submultiples of Basic Supplementary and Derived Units of Measurement	12

Schedule II	13

Canadian Units of Measurement	13

Insert a copy of the Weights & Measures Act.(PDF)

###### Rationale

## Section 2

### Definitions

Local Standards designated by the Minister have been calibrated and certified in relation to a reference standard, the accuracy of which will be within the prescribed tolerances.

A machine designed specifically to count items is not included in the definition of “measuring machine” and is not a device as defined by the Act. As such, this type of device is not subject to the Weights and Measurements Act and Regulations. A device that is capable of both counting and weighing items must have its weighing functions approved and inspected if it is to be used in trade.

## Section 3

### Approval of Devices by the Minister

For all devices that are conditionally approved under Subsection 3(2), inspectors will ensure that the proper disclosure notifications are met prior to a device being examined and before its first use in trade (initial inspection).

Disclosure notifications are to be established before concluding the sale of a conditionally approved device. The seller of the device must make known to the purchaser the following information:

final approval is contingent on the results of the onsite examination, and

any necessary modifications will be made by the approval applicant.

Review Bulletin GEN-6

## Section 8

### Use of Devices

Section 8 forms the fundamental requirements for traders to possess and use in trade only specific devices that:

have been approved for their intended use in trade, and

have been examined prior to their first use in trade

In reference to Section 3, it grants the Minister authority for approval.

## Section 10

### Regulations

The Governor in Council may make regulations respecting:

Application for approval

Conditional Temporary Approval

Temperature scales

Inspectors’ duties

Commodity Acceptance Criterion

Inspection circumstances

Reporting requirements

Locations

Device Repair/Alteration

Governor in Council may make regulations regarding:

Establishment of specifications

Device suitability for trade

Seizure and detention of devices and commodity

Disposition and forfeitures of devices and commodity

Authorizing units of measurement

Advertising of units of measure price per unit of mass

Governor in Council may make regulations regarding:

Documentation

Inspector fees

Related examination fees

Marking of coin operated liquid vending machines

Device inspections

Anything else that can be established under the Act

## Section 13

### Reference and Local Standards

All District Managers and Specialist positions are formally delegated the authority to designate local standards respective of their positions. Designation of local or working standards is restricted to the scope of the particular specialists duties and responsibilities.  That is to say, Gravimetric specialists may designate mass standards but not volume standards.

Local Standards are certified within periods established by Section 56 of the regulations.

## Section 15

### Examination of Devices

Section 15(1) requires traders to have scales examined within prescribed periods.

15(2) gives, the minister, the ability to grant restricted extension periods.

Authority to make adjustments with the consent of the device owner is granted here in Section 16. It is important to make adjustments with great caution as you will be responsible and may be liable for your adjustments.

## Sections 19

### Certificate and marking

In section 19 the term “examines” refers to the actions of an inspector with respect to a device, including visual examination, testing and marking.

Generally, when an inspector examines a device that is approved and used in trade the inspector will issue a certificate.

## Section 23

### Offences and Punishment

Section 23 makes it an offence for any one to use a device in trade that is prohibited by:

Annotations at the time of last examination.

Specifics written in the notice of approval for that device.

Part of an inspector’s responsibility is to review past inspection documents, device history reports and approvals for such annotations or restrictions placed on a device at either its approval or first examination (s) prior to use in trade. Annotations usually appear in the last line of a history report you will generate or can have generated.

## Section 24

### Offences and Punishment

Section 24 of the Act states that devices used in trade have to be installed in accordance to the requirements and they have to measure within the limits of error that are specified by the regulations.

The regulations will specify the two types of error limits:

Acceptance limits of error, and

In-service limits of error

## Section 25

### Examination within prescribed period

This section creates an offence if the trader fails to have his device examined within the prescribed periods.

## Section 26

### Offences and Punishment

Section 26 requires device dealers to have a device examination and/or marked appropriately before it is sold, leased or disposed of.

“Not for use in trade until examined”

“Not Legal for Trade”

“Not for use in Trade”

## Section 28

### Offences and Punishment

Under Section 28, a person is guilty of an offence if they allow a ticket to remain in the printer at any other time than when the commodity is being measured. For example:

A liquid delivery meter mounted on a truck is prohibited from traveling between two establishments with the ticket remaining in the printer. (This ticket may have been initiated during a previous delivery).

Drivers are prohibited to leave the delivery site without resetting the meter register to zero since the next customer may be invoiced for a larger quantity of product than what was actually delivered. (Meter registering printers indicate total volume by subtraction of the start and finish print.) However, a delivery ticket may remain in the printer if the person is filling multiple tanks at one establishment provided the vehicle is not in motion on a public street or highway between deliveries.

Every person who has accepted and repaired a device on behalf of a trader under Section 29 is required:

To have the device either examined by an inspector and marked appropriately, or

Make a report in writing to an inspector regarding the repairs or alterations to a device. To ease reporting requirements as per Section 41 of the Regulations. Measurement Canada has developed and does issue companies with preprinted forms called the “short form”.

## Section 30

### Offences and Punishment

Section 30 makes it an offence for anyone to remove any marking or break any seals which were placed on a device by an inspector or person who had repaired, calibrated or altered a device.

Subsection 2 allows the removal of markings or breaking of seals if a person has the intent of repairing or calibrating a device and a report describing the removal, repair or adjustment is made and sent to the nearest Measurement Canada Office within 5 days of the date of the adjustment, repair or removal of markings.

## Section 31

### Offences and Punishment

Section 31 makes it an offence for anyone to:

obstruct an Inspector performing his duties

knowingly makes false or misleading statements to an inspector

removes, alters or interferes with any device, commodity or other thing seized by an inspector without their consent

## Section 35

### Offences and Punishment

Section 35 prescribes the maximum punishment for anyone found guilty under the various sections of this legislation.

Criminal offences are tried in the courts according to one of two methods of procedure: a) indictment, or b) summary conviction. The existence of these two methods permits some flexibility in the handling of offences of varying magnitude. Indictment, the standard procedure for serious crimes, (fraud) is usually a lengthy process which provides the accused with several safeguards, most notably, trial by jury. Summary conviction, as its name suggests, is a much faster procedure. The law has reserved the summary conviction procedure in general to offences which carry a lesser punishment.

## Section 36

### Evidence

Under Section 36 an inspector will assume that every device is held for use in trade unless the inspector can establish visual evidence to the contrary.

“Not Legal for Trade”

“Not for use in Trade”

## Section 37

In Section 37 an accused individual may not be found guilty if he can establish “due diligence”. In other words, the individual did everything in his/her power to try and prevent the offence from occurring. The accused could argue something resembling: “the offence was committed by another person, a member of my staff. That person had the appropriate training and was given all the right tools. He was told and continually reminded not to commit the offence. The offence occurred, but I can’t be everywhere at once! I did everything in my power to ensure the offence would not happen! What more could I do?”.

## Schedule 1

(Sections 2. 4, 6, 7, 10 and 12)

### Part I – Units Based on the International System of Units

|  | Basic Unit | Symbol | Definition |
| --- | --- | --- | --- |
| 1 | metre | m | the unit for the measurement of length, being the metre as defined by the 26e Conférence générale des poids et mesures as reported in the Comptes rendus de la 26e Conférence générale des poids et mesures published by the Bureau international des poids et mesures, as amended from time to time |
| 2 | kilogram | kg | the unit for the measurement of mass, being the kilogram as defined by the 26e Conférence générale des poids et mesures as reported in the Comptes rendus de la 26e Conférence générale des poids et mesures published by the Bureau international des poids et mesures, as amended from time to time |
| 3 | second | s | the unit for the measurement of time, being the second as defined by the 26e Conférence générale des poids et mesures as reported in the Comptes rendus de la 26e Conférence générale des poids et mesures published by the Bureau international des poids et mesures, as amended from time to time |
| 4 | ampere | A | the unit for the measurement of electric current, being the ampere as defined by the 26e Conférence générale des poids et mesures as reported in the Comptes rendus de la 26e Conférence générale des poids et mesures published by the Bureau international des poids et mesures, as amended from time to time |
| 5 | kelvin | K | the unit for the measurement of thermodynamic temperature, being the kelvin as defined by the 26e Conférence générale des poids et mesures as reported in the Comptes rendus de la 26e Conférence générale des poids et mesures published by the Bureau international des poids et mesures, as amended from time to time |
| 6 | candela | cd | the unit for the measurement of luminous intensity, being the candela as defined by the 26e Conférence générale des poids et mesures as reported in the Comptes rendus de la 26e Conférence générale des poids et mesures published by the Bureau international des poids et mesures, as amended from time to time |
| 7 | mole | mol | the unit for the measurement of the amount of substance, being the mole as defined by the 26e Conférence générale des poids et mesures as reported in the Comptes rendus de la 26e Conférence générale des poids et mesures published by the Bureau international des poids et mesures, as amended from time to time |

### Part IV – Customary Units of Measurement with the International System

|  | Customary Unit | Symbol | Definition |
| --- | --- | --- | --- |
| 1 | Minute | min | 60 seconds |
| 2 | Hour | h | 3 600 seconds |
| 3 | Day | d | 86 400 seconds |
| 4 | Degree (of arc) | ° | π/180 radian* |
| 5 | Minute (of arc) | ‘ | π/10 800 radian* |
| 6 | Second (of arc) | “ | π/648 000 radian* |
| 7 | Tonne or metric tonne | t | 1 000 kilograms |
| 8 | Litre | L. l or l | 1/1 000 cubic metre |
| 9 | Milliliter | mL, ml or ml | 1/1 000 litre |
| 10 | Hectare | ha | 104 square meters |
| 11 | Degree Celsius | °C | An interval of 1°C = an interval of 1 kelvin; a temperature of 0°C corresponds to 273.15 kelvins |

### Part V – Prefixes* for Multiples and Submultiples of Basic Supplementary and Derived Units of Measurement

|  | Prefix | Symbol | Definition |
| --- | --- | --- | --- |
| 1 | exa | E | 1018 |
| 2 | peta | P | 1015 |
| 3 | tera | T | 1012 |
| 4 | giga | G | 109 |
| 5 | mega | M | 106 |
| 6 | kilo | k | 103 |
| 7 | hecto | h | 102 |
| 8 | deca | da | 101 |
| 9 | deci | d | 10-1 |
| 10 | centi | c | 10-2 |
| 11 | milli | m | 10-3 |
| 12 | micro | μ | 10-6 |
| 13 | nano | n | 10-9 |
| 14 | pico | p | 10-12 |
| 15 | femto | f | 10-15 |
| 16 | atto | a | 10-18 |

* not applicable to the basic unit “kilogram” but applicable to the one thousandth submultiple thereof, namely the “gram”.

## Schedule II

(Sections 2. 4, 6, 7, 10 and 12)

### Canadian Units of Measurement

| Measurement of Length |  |  |
| --- | --- | --- |
|  | Unit of Measurement | Definition |
| 1 | mile | 1 760 yards |
| 2 | furlong | 220 yards |
| 3 | rod, pole or perch | 5½ yards |
| 4 | yard | 9 144/10 000 metres |
| 5 | foot | 1/3 yard |
| 6 | inch | 1/36 yard |
| 7 | chain | 22 yards |
| 8 | link | 1/100 chain |

| Measurement of Volume or Capacity |  |  |
| --- | --- | --- |
|  | Unit of Measurement | Definition |
| 1 | bushel | 8 gallons |
| 2 | peck | 2 gallons |
| 3 | gallon | 454 609/100 000 000 cubic metre |
| 4 | quart | 1/4 gallon |
| 5 | pint | 1/8 gallon |
| 6 | gill | 1/32 gallon |
| 7 | fluid ounce | 1/160 gallon |
| 8 | fluid dram | 1/8 fluid ounce |
| 9 | cubic yard | A volume equal to that of a cube each side of which measures one yard |
| 10 | cubic foot | 1/27 cubic yard |
| 11 | cubic inch | 1/1 728 cubic foot |
| 12 | revoked | Statutory Order and Regulations (SOR)/86-854 |

| Measurement of Mass or Weight |  |  |
| --- | --- | --- |
|  | Unit of Measurement | Definition |
| 1 | ton | 2 000 pounds |
| 2 | central or hundredweight | 100 pounds |
| 3 | pound | 45 359 237/100 000 000 kg |
| 4 | ounce | 1/16 pound or 437½ grain |
| 5 | dram | 1/16 ounce |
| 6 | grain | 1/7 000 pound |