Bulk and Retail Petroleum

# Module 6

# Specifications

January 2021

Uncontrolled copy for training purposes only

Table of contents

SVM-1 Rationale	4

Definitions	4

Application	4

Pulsers	4

Pulse Processing	5

Controls and Interlocks	6

Registrations	7

Non-resettable Registers	7

Printers	8

Performance	9

Installation and Use	10

SVM-2 Rationale	12

Definition	12

Exception	12

Design, Composition and Construction	12

Performance	15

Installation and Use	16

SVM-1: Specifications on electronic registers and ancillary equipment incorporated in metering assemblies’ specifications (PDF)

# SVM-1 Rationale

## Definitions

“Non-resettable Register”: This term refers to a primary register that is intended for long-term use without being reset to zero. An example of a non-resettable register would be a register installed on a metering assembly attached to a house for use in monitoring the consumption of fuel oil into the house’s oil furnace heating system. Other types of non-resettable registers can be found on liquid pipeline installations.

“Register”: The definition of register does not include the cash register’s console, which might also register the volume and total price as a means of secondary indication of the metering assembly (e.g. fuel dispenser).

## Application

Section 3 notes that the following specification applies to electronic registers only.

## Pulsers

Section 4 specifies that pulsers are to be designed in order to perform within the same limits of error as registers when subjected to:

a temperature range of -30° to + 40° C.

a humidity range of 10% - 95%.

90 - 110% of specified nominal power requirements

voltage spikes of 10/sec for 10 minutes.

radio frequency interference @ 27MHz and 460 MHz at 2 m with 4w transceivers.

Section 5 ensures that electronic registers are capable of recognizing false or missing pulses.

Section 6 specifies that metering assemblies must either have a positive means to eliminate reverse flow or the electronic register must be capable of distinguishing direction of flow.

## Pulse Processing

Section 7 ensures that registers are required to detect false or missing pulses.

Section 8 notes that if a pulse processing system’s design permits pulses to be adjusted or altered, then, mechanism must be sealable and not readily accessible without removal of some portion of the register case.

Section 9 notes that any readily accessible means of adjustment (calibrating mechanism) must not function while the meter is operating, must be sealable and is limited to a range of ± 2% of the delivered volume.

Section 10 explains that in certain circumstances, registers may incorporate a means of adjustment that permits to linearize the meter’s accuracy/flow rate performance curve. In fact, the actual quantity delivered at the slow flow for a positive displacement meter can be substantially higher than at fast flow. Electronic register manufacturers recognize this and can design equipment that automatically changes the meter calibration at different flow rates to enhance meter performance.

Figure 1 – Curve showing the meter error versus the flow rate

A meter factor = prover volume/meter registration

= 20.140 / 20.000

= 1.007

If the uncorrected meter registration is multiplied by the meter factor of 1.007, the calibrated meter registration of 20,140 will agree with the actual volume.

The programmed meter factors must be verifiable.

Section 11 specifies that:

1)	If reverse flow is possible, the amount of reverse flow shall be subtracted from the total quantity delivered.

2)	Where a system is not capable of distinguishing the difference between forward and reverse flow, the system will be restricted to installations where reverse flow cannot occur. The system must be installed with a back check valve to stop reverse flow.

Section 12 specifies that registers are to be designed to allow an operator to detect any fault in a segment or in a circuit of the segment or to perform automatic segment checks of its displays. Where faults are detected it will prevent delivery, e.g. solenoid valve.

## Controls and Interlocks

Section 13 prohibits a unit price change while the register is in use.

Section 14 states that registers are to be designed to automatically stop the flow of product and cease registering when false or missing pulses are detected greater than 5 X the finest increment of registration and 1 in 1000 increments.

increment of registration = 0.001 
After an amount of false or missing pulses are detected equal to 5ml and/or 1ml in 1000ml, the register display should lock and stop the flow of product.

Section 15 allows a reasonable time for the user to reconcile details of the transaction with, for example, the cash register console at the point of sale.

Section 16 specifies that while registers can be designed to operate with multiple meters or a single meter and multiple products, their means for adjustment must correspond to the product or the meter necessary of the adjustment.

Section 17 notes that registers that are designed to operate with multiple meters or a single meter and multiple products must prevent the selection mechanism from operating after 10 times the smallest increment of registration has been achieved unless the register automatically resets to zero on selection.

Section 18 specifies that registers are restricted to the delivery of only one product at a time.

Section 19 in order to hide the effects of hose expansion to the maximum allowance of 3 minimum increments of registration, allows for one of 2 options (for additional information consult STP24):

Registration

Delivery

0.00

0.00

0.00

0.00

0.01

0.031 litres

0.02

Registration

Delivery

0.00

0.00

0.00

0.00

0.01

0.031 litres

0.02

0.01 litres

Suppression

Masking

0.031 litres

Figure 2 – Masking and suppression are the two options allowed in order to hide the effects of hose expansion

## Registrations

Section 20 authorizes the minimum increment of electronic registration.

Review Bulletin V-7 for requirements regarding the finest increment of registration for metering assemblies and associated printing devices.

Section 21 specifies that a register that incorporates total price must have a sufficient number of digits to display the largest delivery volume and unit price possible.

## Non-resettable Registers

Section 22 specifies that, when used with a meter such as a pipeline meter, a non-resettable register will be designed so that no registration is repeated when the meter operates continuously at its maximum rated capacity for a period of 90 days. Moreover, a non-resettable register must have a sufficient number of digits to display the results of a meter operating continuously and at capacity (L/min) for 90 days.

Section 23 requires a non-resettable register being exposed to outdoor conditions to incorporate, at the very least, a 24 hour back-up power supply on failure of the main power where the back-up power supply is re-charged via the main power supply or 7 days of back-up power when any other type of back-up power supply is used.

Section 24 requires periodically replaced power sources to indicated low power at least 90 days before any power failure. The replacement of this source cannot alter the stored register information, calibration etc.

Section 25 sets out requirements to protect registers (other than non-resettable registers) from power failure and voltage drop (brown-out) to the same extent as Section 23. Additionally registers must be capable of displaying the quantity, unit price and total price for a period of 15 minutes after power failure.

Section 26 requires that registers designed for measuring flow in both directions through a metering assembly indicate the direction of flow.

## Printers

Section 27 sets out requirements for meter assembly printers. (Not consoles or cash registers). Printers must provide the following printed record:

Name and address of the trader

Installation address or vehicle identification

Identification number of meter where more than 1 meter

Date of the delivery.

In conjunction with Section 27, Regulation 129 requires:

the quantity or start and finish print

if computed price, the unit price

the unit of measure and,  if applicable the unit price and total price.

## Performance

Section 28 specifies that when the total price is calculated by the register, the unit price and the quantity indication must agree to the nearest cent.

Section 29 ensures a register and its equipment will be designed and constructed for its intended environment as per the temperature and humidity range stated.

Section 30 requires a register and its equipment to perform over the stated range of the nominal.  This testing should be left for the approval laboratories.

Section 31 examines the requirement concerning tests where registers are subjected to voltage spikes. It is important to note that radio frequency interference to 400MHz is also a laboratory test.

Section 32 This section forms the basis for testing a device’s preset volume controls.

For example, when the register is preset for 10.00 litres, the dispenser must deliver at least 10.00 litres, but not more than 10.05 litres.

Section 33 intends to control any inaccuracy that may occur because of hose expansion hose softness) and/or defects in the anti-drain valve. This section forms the basis for testing a device’s preset money controls.

For example, when the register is preset for $ 5.00, the dispenser must stop delivery at $ 5.00's worth, but not anymore than $ 5.00 + ( 0.05 litres X price/litre).

For instance: fuel sells for $1.03 / litre.

$1.03 X 50 ml = 5.15 cents

rounded up = 6 cent tolerance.

## Installation and Use

Section 34 specifies that meters not intended to register reverse flow must incorporate a means to prevent this phenomenon from taking place. A back-check valve installed downstream of the metre or an electric solenoid valve are both positive means.

Section 35 insures that any backlash between the measuring element and the pulser does not exceed the motion required to generate pulses corresponding to the applicable minimum increment of registration set out in Section 20. This is done by requiring the installation of a pulser in the metering assembly. In fact, excessive play between the measuring chamber and the pulser might cause a pulser to chatter back and forth. This motion could ultimately create phantom pulses while no product is actually being measured. A field test for this condition is called the anti-backlash test.

Section 36 requires that, in addition to the printed information required by Section 27, a register with a money acceptor must also print the amount of money tendered and the amount of change given back.

Section 37 requires that a metering station be able to stop the liquid’s flow when false or missing pulses are detected that represent an amount of liquid exceeding the limit determined unacceptable by Section 14. The phrase “Automatic means to stop the flow of liquid” generally suggests the use of a solenoid valve or the electrical switching off of the pumping motor.

Section 38 specifies that a metering installation that incorporates a register (as referred to in Section 18) needs to insure by automatic means that not more than one liquid is delivered at one time. It is important to remember that manual valves are not considered automatic means. Generally, electrical solenoid valves are incorporated.

SVM-2: 	Specifications on metering assemblies incorporating electronic ATC’s (PDF)

# SVM-2 Rationale

## Definition

The acronym “VCF” stands for volume correction factor. It is defined as such in Section 270 of the Regulations, which authorizes the use of the 1980 edition of the API/ASTM tables for volume correction factors for all petroleum liquids except liquid propane. The original 1953 edition of the ASTM/IP tables (metric edition) is authorized for this product.

Section 3 - these specifications apply to all metering assemblies incorporating ATCs of the electronic type.

## Exception

Section 4 authorizes meters equipped with automatic temperature compensation (ATC) to register the volume of liquids in units corrected to 15̊ C and to the reference pressure when it comes to liquid petroleum products, liquified petroleum gases, butane and natural gases.

## Design, Composition and Construction

Section 5 ensures that where a meter register has automatic temperature compensation (ATC), the net volume is the basis of the transaction. The gross volume is used by the inspector to calculate the volume correction factor (VCF) applied by the automatic temperature compensation (ATC) from the ratio of net to gross; therefore, it is necessary that the gross volume be determined directly from meter output.

Reference: Section 5.7 of Bulletin V-29 Legislative rulings and interpretations

Section 6's purpose is to ensure that the total price is always calculated the net volume. Otherwise, a trader may be inclined to calculate it from the net with liquid temperatures below 15°C and from the gross with the temperatures above 15°C.

Section 7's requirement prevents unauthorized tampering with the automatic temperature compensator (ATC) setting or, if applicable, the circuit board containing the automatic temperature compensator’s (ATC) chips, etc. In the latter instance, sealing the cover providing access to the circuit board would be adequate - the board would not have to be sealed in place.

Section 8 the temperature sensor is used to indicate when the temperature is stable enough for a test of the automatic temperature compensator (ATC) and to monitor product temperature during the test.

Section 9 ensures that when a sensor is replaced, the automatic temperature compensator (ATC) will be accurate without an adjustment of the temperature measuring circuit.

Section 10 prevents tampering with the sensor, such as withdrawing it from the well and exposing it to ambient temperature.

Section 11 prevents the use of the register if the circuit to the sensor is damaged or tampered with.

Section 12 permits meters that measure various liquids, or meters that each measure a liquid, to have a register with automatic temperature compensation–on condition that there is an interlock to select the proper temperature correction factors for the different liquids metered.

Section 13 specifies that a register incorporating an automatic temperature compensator (ATC) will be designed so that it indicates the density/cubical coefficient of thermal expansion per degree Celsius of the liquid for which it is designed to be used. If it is designed to be used for more than one liquid, it will indicate the range of densities or cubical coefficients of thermal expansion per degree Celsius to which it is adjusted by the means enumerated in the section. An inspector cannot verify or reject a register with automatic temperature compensation (ATC) unless it is marked with information which permits the inspector to determine which table of volume correction factors should be used. While most registers will be marked with the density, some (for use on special liquids) may be marked with the coefficient of cubical expansion, found in API/ASTM Table 54C. The Accuload and similar registers, which are programmed after installation for the required density, will display the set density on demand. Where an automatic temperature compensator (ATC) is to be used for only a few fixed densities (e.g. those for only gasoline or fuel oil), density selection should be made as specified in 13(d) or 13(e).

Section 14 explains that in addition to the information required by Section 27 of SVM-1 concerning The Electronic Register and Ancillary Equipment Incorporated in Metering Assembling Specifications, a printer forming part of a metering assembly that indicates the amount of a delivery from a bulk meter, receiving meter or aircraft refueller equipped with an automatic temperature compensation will provide a ticket that shows:

the net volume;

the gross volume;

the density used to determine the volume correction factor (VCF).

There are mechanical ticket printers available (in Europe) that advance the ticket (in the manner of a cash register tape) so that more than one delivery can be printed on the same ticket. This same principle could be applied to print net, gross and density values. This requirement is necessary so that traders not using automatic temperature compensation can reconcile their input/output and so that aircraft flight engineers can use the value to which they are accustomed.

Must print density of the product or the generic product name “JET B”.

Section 15 - a thermal well that does not closely fit the sensor or that is too thick, or of a material that will be corroded by the liquid measured, can slow down the response of the sensor to temperature change, resulting in inaccurate compensation. Therefore, well/sensor assemblies shall be tested as required by section 18 and the well sensor identified in a Notice of Approval. The requirement to have the name or trademark of the manufacturer and the model number of the well on the end fitting is not currently enforced.

Section 16 requires of an automatic temperature compensator designed for a meter that measures liquids at an ambient temperature that it corrects the measurement results within the applicable limits of error set out in Section 270 of the Regulations over at least the range of liquid temperature of -30̊ C to 40 ̊ C. This temperature range is typical for liquids in Canada. However, if the name plate of the automatic temperature compensator indicates a wider temperature range, the correction must be made over at least that wider temperature range.

Please consult Section 29 in SVM-1 for correct operations at ambient temperatures.

Section 17 explains that an automatic temperature compensator designed for a meter that measures liquids at a temperature different than ambient needs to correct the measurement results within the applicable limits of error set out in Section 270 over at the temperature range marked on the name-plate of the automatic temperature compensator. This range will cover at least 50 degrees Celsius. Yet, in the interest of better accuracy in temperature measurement/compensation, it is advantageous to limit the automatic temperature compensator’s range. Too narrow a range is not acceptable as the liquid temperature then may fall outside this range.

Section 18 requires that when a liquid’s temperature is outside the automatic temperature compensator’s temperature range, the ATC will automatically prevent further deliveries or remain operative where the VCF does not exceed the value corresponding to the temperature of the liquid. Therefore, paragraph (b ) of this section permits an ATC error against the trader in excess of the prescribed limit of error for temperatures below -30°C for liquid propane, etc. One of the problems here is that the API-ASTM tables for gasoline, fuel oil, etc., only go to -18°C.

## Performance

Section 19 sets the performance requirements concerning time response for test liquids and temperature sensors in metering assemblies containing automatic temperature compensation. The mean velocity of the liquid stream (calculated from the pipe area and flow rate) is used rather than flow rate, as the use of the latter would require a table of values of flow rate against pipe size. A substantial change in temperature is specified so that any errors in measuring the temperature are insignificant in relation to the total change e.g. an error of 0.5°C in 20°C is not as serious as 0.5°C in 5°C. Measuring the time required for 90% of response to the step change is an industry standard. A shorter response time is specified for motor fuel dispensers than for other meters because bare probes can usually be safely used in this application. Moreover, a short response time is important for motor fuel dispenser since, unlike automatic temperature compensator’s on truck-mounted fuel oil meters, they are often used for deliveries of less than a minute.

Section 20 requires that a meter equipped with an automatic temperature compensator (ATC) meets the performance specifications applicable to volumetric liquid meters when inspected using the gross mode of registration. This requirement is necessary because the volumetric accuracy of the meter is tested independently of the automatic temperature compensator (ATC). In some countries, the net registration alone is tested.

Section 21 specifies that an automatic temperature compensator (ATC) will stay within the applicable limits of error set out in Section 270 of the Regulations when the automatic temperature compensator (ATC) and sensor are tested under the conditions set out in Sections 29 to 31 of SVM-1.

Refer to Regulation 270 for limits of error for mechanical and electronic automatic temperature compensation (ATC).

Section 22 states that the test volume for an automatic temperature compensator (ATC) will be the same as a normal delivery through the meter. However, the test volume shall never be less than 2, 000 times the value of the minimum increment of registration of the meter register (as set out in section 20 of SVM-1). This minimum test volume is necessary in order to make the error in reading (rounding off error) of the register negligible. As standard electronic register accuracy is ±1 times the smallest digit, the reading error is 2000 increments is 1 in 2000 or 0.05%.

## Installation and Use

Section 23 (a) requires that the sensing end of the sensor be located where the liquid velocity is greater (i.e. near the axis of the pipe). This ensures good speed of response while permitting the sensor’s end to be distanced from the pipe wall since the latter could be hotter or colder than the liquid flowing during the delivery.

Section 23 (b) - requires that no valves, pumps or other equipment that can cause a change in temperature of the flowing liquid be installed between the sensor and the meter. This ensures the sensor senses the same temperature as the meter.

Section 24 explains that adjacent to a temperature sensor being used with an automatic temperature compensator (ATC), there should be installed a test thermal well that has an inside such that only a small quantity of heat-conducting fluid is required to fill the void. (i.e: diameter of no less than 3.3 mm and no more than 4.5 mm.)

Section 25's intention is to minimize the transfer of heat from the sensing end of the sensor to the atmosphere. Large aluminium electrical connection boxes that readily dissipate the heat conducted by the well from liquid to atmosphere is an example of this. In a case such as this, the liquid sensor quickly reaches ambient temperature.

Section 26 A trader often has no control or knowledge of the density of the liquid being metered. For example, to permit the use of ATC on dispensers for gasoline, an average value of .730 kg/litre was selected, and ATCs were designed for this density. This section, by allowing use of the ATC on gasolines, of which VCF does not differ by more than 0.25% at 0°C from that of the average gasoline, permits the ATC to be used on gasolines with densities of from .672 to .776 kg/litre (at 15°C).

For an ATC designed for a density of 0.730 kg/litre at 15°C, the VCF at 0°C = 1.0187

1.0187 + 0.25% = 1.0212 which is the VCF at 0°C for a product density of 0.672 kg/litre at 15°C

1.0187 - 0.25% = 1.0162 which is the VCF at 0°C for a product density of 0.776 kg/litre at 15°C

The limits are therefore: 0.672 kg/litre and 0.776 kg/litre

Section 27 insures that where one of the meters in a metering assembly used for delivering a blend of liquids is equipped with an automatic temperature compensator (ATC), all the other meters in the metering assembly shall also be equipped with automatic temperature compensators (ATCs).

Some blending systems were being installed with automatic temperature compensation (ATC) only on the meter(s) for hot liquids, which was an advantage to the trader. In the interest of equity in trade, this section also requires automatic temperature compensation (ATC) on cold liquids, where it may be to the advantage of the buyer.

Section 28 states that when a meter has means of adjustment, these will be installed in such a manner that they are applied to both the net and the gross registrations.

API/ASTM stands for American Petroleum Institute/American Society for Testing Materials.

ASTM/IP stands for American Society for Testing Materials/Institute of Petroleum